EXPO PRESTIGE
TECHNICAL NOTES

Purpose: Organize the handwritten observations into a readable reference for planning, designing, and documenting Expo Prestige modular exhibition stands.

# 1. Structure and Configuration

Expo Prestige modular exhibition system is available in the following standard sizes/ configurations.

  - 3 × 1 - W 780 x H 2367 MM

  - 3 × 2 - W 1560 x H 2367 MM

  - 3 × 3 - W 2340 x H 2367 MM

  - 3 × 4 - W 3120 x H 2367 MM

The structure configuration should be planned according to the required booth size/ Floor plan

Booth/ space open sides configurations needs to be define -1, 2, 3, and 4-side open.

Obtain the required floor plan/layout or reference from the client.

Structure sizes and configuration / dimensions should be mentioned against the Top plan/ Layout.

# 2. Connectors and Structural Accessories

Connector options include straight connectors and curved connectors for different structural configurations.

Straight connectors – 190mm, 445mm & 780mm

90° inside & Outside Connectors: Used to create corner configurations such as L-shaped and U-shaped booths.

45° inside & Outside Connectors: Used to create 45° angled configurations.

Strut End Cap and Flat end caps are used to cover structure from the sides so that structure hardware will not be visible from sides.

Additional accessories may be required to properly close or finish open sections of the structure.

Stabilization foot are required, each structure required two foots. It’s a standard size.

Structure height is standards – 2367mm.

Magnetic Straight Strut are part of the system.

# 3. Graphics, Panels, and Branding

Digital vinyl print with lamination pasted on the PVC sheets and at the back side there is magnets.

Male/female Magnetic struts are used for attaching prints to the structure

Top branding panel sizes is standards – (1x1- W 780mm x H780mm and 1x2 – W 1560mm x H 780mm)

# 4. Counter and Display Podiums

A. Reception Counter:

- Butterfly Counter

- Rapid Counter

2. Product Podiums / Counters:

- Round Counter

- Hype Podium

- Eurofold Counter

5. Display and Functional Components

Window showcase size is standard and fixed, and used for product display

One window showcase is having two shelves and two spot lights for product illumination.

Clear acrylic sheet is used as the standard front cover for the window showcase.

Reception table: foldable and reusable design.

42” TV screen can be fixed to the same structure.

Product display podium/counter: include a podium or counter for presenting/ showcase the products.

Brochure stand: A4-size brochure/catalogue display.

Store room to be used to store the empty boxes and other required materials.

Open meeting area: appropriate seating arrangements.

- Standard reference configuration:

- 1 round table

- 4 chairs

# 5. Space Planning and Flooring

The complete stand footprint should be carefully planned to ensure that elements fits within the allocated exhibition space/ Floor plan.

Flooring: Carpet finish is noted; coordinate carpet design and floor dimensions with the layout.

Before finalizing the design, confirm:

- Overall booth dimensions/ Requirements

- Branding requirements

- Open sides/access points

- Furniture requirements

- TV/ AV Screens requirements

- Display positions

- Storage requirements

- Meeting space positioning

# 6. Top Plan and Layout

Prepare a top-plan and elevation plan indicating the overall booth dimensions – width, length and height.

Mentioned the total booth area in meters.

The top plan should clearly indicate the placement of:

- Overall booth/ space-  width, length and height

- All the structures

- Reception/counters

- Meeting areas

- Storage

- Product displays

- Other functional components

# 6. Standard Brief Form

- A standard brief form / template need to be developed to capture all the project information required for designing.